CHECK LIST BEFORE LEAVING THE BOAT

- Check shackles have been moused with cable ties (if on mooring).

- Shut down engines.

- Raise engines.

- Check fuel in tanks – is there enough for next time?

- Close breather vents in tanks.

- Turn off and remove engine circuit breaker keys and put them in top pocket of tool box.

- Switch off all instruments on panel.

- Turn off house battery circuit breaker key (leave key hanging on cord).

- Replace rubber Raymarine cover on plotter.

- Close side window (if open).

- Remove ignition keys.

- Check all flags have been lowered and stowed.

- Return life ring to cabin.

- Return yellow buoy (from main mooring rope) to cabin.

- Lock cabin and toilet doors.

- Wash down decks and return brush and bucket to fish locker.

- Lock fish locker.

- Ensure gunwhale door is securely closed.

Please report any faults/issues to Colin Smith - 07742 783085